Annex H

Comprehensive study of ASPPs inverters technical capability grouped by manufacturers

Manufacture FOX-ESS

All modern Fox-ESS inverters (F, G, T, H3, H3-Pro, etc.) have dynamic voltage control functions.

- F Series (G2):

  - FOX F5000-G2: Single-phase, 5kW inverter with 2 MPPTs.

  - FOX F6000-G2: Single-phase, 6kW inverter with 2 MPPTs.

- G Series:

  - FOX G8-8.000W: Single-phase, 8kW inverter with 3 MPPTs.

  - FOX G10-10.000W: Single-phase, 10kW inverter with 3 MPPTs.

- T Series (G3):

  - FOX T10-10.000W: Three-phase, 10kW inverter with 2 MPPTs.

They support:

- Grid codes (EU, UK, Australia, US, ENTSO-E, NERC, etc.) require inverters above a few kW to support:

  - Reactive power control vs. voltage (Q(U))

  - Active power vs. frequency (P(f))

  - LVRT / HVRT (fault ride-through)

  - Adjustable power factor

- Fox-ESS datasheets and manuals (for F-Series, G-Series, T-Series, H3 / H3-Pro, etc.) all explicitly mention:

  - Power factor adjustable (0.8 leading – 0.8 lagging)

  - Reactive power control (dynamic)

  - Volt-var and Volt-watt control modes

- Hybrid models (H3, H3-Pro, H3-Pro-30, KH, EP series) additionally provide voltage regulation in backup/EPS mode. They actually form the local grid, so they are true voltage-source inverters when off-grid.

Key difference

- Grid-following inverters (string types: F, G, T, etc.): only support grid voltage by injecting/absorbing reactive power. They cannot set the grid voltage themselves.

- Hybrid / off-grid capable inverters (H3, H3-Pro, EP, etc.): can also form and regulate voltage in backup/micro grid mode.

So:

- All models in our list have dynamic voltage control when grid-tied (through reactive power injection/absorption).

- Hybrid inverters (H, H3, KH, H3-Pro, EP, some G-series etc.) support full DVC, plus provide stand-alone voltage regulation in off-grid mode. So they can do voltage/frequency regulation (they can act as voltage sources in backup/EPS mode).

- String/grid-tied inverters (non-hybrid): F-series, G-series (standard), T-series, V-series, R-series, S-series (F, G, V, R, S) support grid-support DVC (reactive/voltage curves), but cannot form a grid by themselves. Their “dynamic voltage control” is limited to what the grid codes allow (no standalone voltage regulation).

- Battery modules (EP11) are not inverters, so no DVC on their own.

- All FoxESS inverters certified for VDE-AR-N 4105 / EN 50549 / G99 support DVC (Q(U), P(f), Volt-Var, etc.). So all models have DVC.

FoxESS Models: Dynamic Voltage Control and Hybrid Capability

| Model | Dynamic Voltage Control | Hybrid (Battery Capable) |
| --- | --- | --- |
| EP 2000 | Yes | Yes |
| EP10 | Yes | Yes |
| F3000 | Yes | No |
| F3600 | Yes | No |
| F5000 | Yes | No |
| F5300 | Yes | No |
| F5300 (14A) | Yes | No |
| F5300-G2 | Yes | No |
| F6000 | Yes | No |
| F6000 (14A) | Yes | No |
| F6000-G2 | Yes | No |
| G10 | Yes | No |
| G10 1 | Yes | No |
| G10.5 | Yes | No |
| G10.5 (1.0) | Yes | No |
| G7 | Yes | No |
| G7, T5, F5300, T6 | Yes | No |
| G8 | Yes | No |
| G8000 (1.0) | Yes | No |
| G9 | Yes | No |
| H1-6.0-E-G2 | Yes | Yes |
| H1-G2-5.0-E | Yes | Yes |
| H1-G2-6.0-E | Yes | Yes |
| H3-10.0-E | Yes | Yes |
| H310E | Yes | Yes |
| H3-12.0-E | Yes | Yes |
| H3-PRO-15.0 | Yes | Yes |
| H3-PRO-20 | Yes | Yes |
| H3-PRO-20.0 | Yes | Yes |
| H3-PRO-30 | Yes | Yes |
| KH10 | Yes | Yes |
| KH10 WITH WIFI | Yes | Yes |
| KH10.5 | Yes | Yes |
| KH8 | Yes | Yes |
| KH9 | Yes | Yes |
| R100 | Yes | No |
| R110 | Yes | No |
| R75 | Yes | No |
| S3000-G2 | Yes | No |
| T10 | Yes | No |
| T10 (G3) | Yes | No |
| T10.5 | Yes | No |
| T10G3 | Yes | No |
| T10-G3 | Yes | No |
| T12 | Yes | No |
| T12-G3 | Yes | No |
| T15 | Yes | No |
| T15 (G3) | Yes | No |
| T15-G3 | Yes | No |
| T17 | Yes | No |
| T20 | Yes | No |
| T20 (G3) | Yes | No |
| T20-G3 | Yes | No |
| T25 | Yes | No |
| T25 (G3) | Yes | No |
| T25-G3 | Yes | No |
| T5 | Yes | No |
| T5-G3 | Yes | No |
| T6 | Yes | No |
| T8 | Yes | No |
| V30 | Yes | No |
| V33 | Yes | No |
| V36 | Yes | No |
| V40 | Yes | No |
| V50 | Yes | No |
| V60 | Yes | No |
| V75 | Yes | No |

Conclusion:

- All FoxESS inverters support Dynamic Voltage Control.

- Hybrid models: EP-series, H1-series, H3-series, H3-Pro, KH-series.

- All other models (F, G, T, V, R, S) are string/standard inverters (non-hybrid).

FoxESS Hybrid Models

- EP Series

  - EP 2000

  - EP10

- H1 Series (Single-phase Hybrid)

  - H1-6.0-E-G2

  - H1-G2-5.0-E

  - H1-G2-6.0-E

- H3 Series (Three-phase Hybrid)

  - H3-10.0-E

  - H310E

  - H3-12.0-E

- H3 Pro Series

  - H3-PRO-15.0

  - H3-PRO-20

  - H3-PRO-20.0

  - H3-PRO-30

- KH Series (High-power Hybrid)

  - KH8

  - KH9

  - KH10

  - KH10 WITH WIFI

  - KH10.5

FoxESS Hybrid Inverters

| Series / Model | Power Rating (kW) | Phase Type |
| --- | --- | --- |
| EP Series |  |  |
| EP 2000 | 2 kW | Single-phase |
| EP10 | 10 kW | Single-phase |
| H1 Series (Hybrid 1-phase) |  |  |
| H1-G2-5.0-E | 5 kW | Single-phase |
| H1-G2-6.0-E | 6 kW | Single-phase |
| H1-6.0-E-G2 | 6 kW | Single-phase |
| H3 Series (Hybrid 3-phase) |  |  |
| H3-10.0-E | 10 kW | Three-phase |
| H310E | 10 kW | Three-phase |
| H3-12.0-E | 12 kW | Three-phase |
| H3 Pro Series (Advanced Hybrid 3-phase) |  |  |
| H3-PRO-15.0 | 15 kW | Three-phase |
| H3-PRO-20 | 20 kW | Three-phase |
| H3-PRO-20.0 | 20 kW | Three-phase |
| H3-PRO-30 | 30 kW | Three-phase |
| KH Series (High-power Hybrid) |  |  |
| KH8 | 8 kW | Three-phase |
| KH9 | 9 kW | Three-phase |
| KH10 | 10 kW | Three-phase |
| KH10 WITH WIFI | 10 kW | Three-phase |
| KH10.5 | 10.5 kW | Three-phase |

Summary:

- Single-phase hybrids: EP & H1 series.

- Three-phase hybrids: H3, H3 Pro, KH series.

- All hybrids support Dynamic Voltage Control.

All FoxESS hybrid and string inverters support P(f) active power-frequency control curve.

- Grid code compliance:
FoxESS inverters are certified under VDE-AR-N 4105, EN 50549, G99, and in some regions IEEE 1547 / UL 1741 SA.
These standards require support for frequency-dependent active power reduction (the P(f) curve, sometimes called frequency-watt or droop control).

- What P(f) curve means:

  - When grid frequency rises above nominal (e.g., from 50 Hz to 50.2 Hz), the inverter must gradually reduce active power output to help stabilize the grid.

  - The slope and deadband are configurable (usually via installer settings or country profile).

- Implementation in FoxESS:

  - All FoxESS F, G, T, V, R, S (string inverters) support P(f) curve.

  - All FoxESS H1, H3, H3-Pro, KH, EP (hybrids) also support P(f) plus additional grid support functions (Q(U) volt-var, Volt-Watt, droop, etc.).

Manufacture SolarEdge

In the collected information exists only two models shown below:

- SolarEdge SE17K

- SolarEdge SE3500H

The result of study is shown below:

- SolarEdge SE17K

  - Dynamic Voltage Control: Yes, supported (as part of compliance with European grid codes such as VDE-AR-N-4105, EN 50549, G99, etc.).

  - Hybrid (battery ready): No, this is a standard three-phase grid-tied inverter, not hybrid.

- SolarEdge SE3500H

  - Dynamic Voltage Control: Yes, supported (same grid code compliance as above).

  - Hybrid (battery ready): Not clearly represented.

An analysis of the main inverter models of SolarEdge show that they generally support standard requiring dynamic voltage control.

Manufacture Sungrow

- SG(40, 50, 125 kW) CX-P2:

These are commercial three-phase inverters, typically designed for grid support and they usually support dynamic voltage/ frequency control (DV/DF), reactive power control, and sometimes LVRT).

- SG(6, 10, 15, 20, 33 kW) RT:

These are residential/small commercial three-phase inverters. Most modern RT series support reactive power control and dynamic voltage regulation, so they also have dynamic control features.

- SG(4.0, 5.0, 6.0 kW) RS:

These are residential single-phase inverters. Depending on firmware, RS models often support static power factor control, but newer versions may support dynamic reactive current injection (LVRT/dynamic voltage).

- (10, 15 kW):

If this is an older model without CX/RT/RS designation, likely it has static control only, no full dynamic voltage/frequency support.

- SG33CX:

CX series (medium power commercial) supports dynamic voltage/reactive control, so yes, dynamic control is supported.

Summary:

- DVC is standard in modern Sungrow string inverters for residential and small commercial installations.

- Older legacy models (pre-2018) might not support full DVC, so it’s always good to check the datasheet.

| Suffix | Meaning |
| --- | --- |
| RS | Residential Single-phase – Designed for residential use with single-phase AC output. |
| RT | Residential Three-phase – Tailored for residential applications requiring three-phase AC output. |
| SH | Hybrid – Capable of both solar energy conversion and battery storage integration. |
| ADA | Advanced – Features multiple MPPTs (Maximum Power Point Trackers) for enhanced energy harvesting. |
| L | Lithium Battery Compatible – Indicates compatibility with lithium-based battery storage systems. |
| CX | Commercial Extreme – High-performance inverters designed for large-scale commercial and industrial applications. |

| Model | Type | DVC Support | Hybrid Capability |
| --- | --- | --- | --- |
| SG5.0RS | Residential Single-Phase | Yes | No |
| SG5.0RT | Residential Three-Phase | Yes | No |
| SG5.0RS-ADA | Residential Single-Phase (Advanced) | Yes | No |
| SH5.0RS | Residential Hybrid | Yes | Yes |
| SG33CX | Commercial Three-Phase | Yes | No |

Example: Sungrow SG20RTHere’s why:

- SG: Sungrow

- 20: 20 kW rated power

- R: Residential / Rooftop

- T: Three-phase

Manufacture SINENG

The Sineng 1P10K and 3P10K inverters are both on-grid models designed for solar energy systems, but they differ in their phase configuration and intended applications.

- Sineng 1P10K (Single-Phase) (no hybrid)

- 3P10K (Three-Phase) (no hybrid)

Summary:

- These inverters do not support hybrid.

- There is no precise information about dynamic voltage control support, but it is likely to be supported, such they support a wide range of standards: IEC 62109-1/2, IEC61000-6, IEC62116, IEC61727, IEC61683, IEC60068, IEC61000-3. EN50549-1/2, VDE4105/4110/4120， C10/11, CEI-0-16/21, BIS/CEA, NRS097-2 KSC8565, TOR Type A&B, PTPiREE , MEA/PEA

Manufacture FRONIUS

| Inverter Model | Dynamic Voltage Control | Hybrid (Battery) |
| --- | --- | --- |
| Fronius 1P4K | Yes | No |
| Fronius 25 | Yes | No |
| Fronius 15 | Yes | No |
| Fronius 5 | Yes | No |
| Fronius 10 | Yes | No |
| Fronius 5 Primo | Yes | No |
| Fronius Symo 7 | Yes | No |
| Fronius 15 Symo | Yes | No |
| Fronius 7 Symo | Yes | No |
| Fronius Symo 10 | Yes | No |
| Fronius Simo 15.0-3-M | Yes | No |
| Fronius 10 3P | Yes | No |
| Fronius 20 | Yes | No |
| Fronius 4 Primo | Yes | No |
| Fronius 3.8 | Yes | No |

- Dynamic Voltage Control: All listed above Fronius grid-tied inverters comply with EN 50549, VDE-AR-N 4105, G99, etc., so they support DVC.

- Hybrid capability: None of these listed models are hybrid.

  - Fronius hybrid-ready models are specifically the Fronius Symo Hybrid (not Symo), or with GEN24 Plus (Primo GEN24, Symo GEN24).

  - The models listed above (Primo, Symo, Simo, standard 1P/3P) are string inverters only.

1. Product family (main name)

- PRIMO: Single-phase inverter for residential rooftop PV systems.

- SYMO: Three-phase inverter for residential or commercial rooftop PV systems.

- ECO: Entry-level inverter for residential PV, usually more basic features.

- SIMO: Sometimes appears as a three-phase residential inverter (less common).

2. Power rating (numbers)

- The number usually indicates AC nominal power in kW, e.g.:

  - Fronius Primo 4: 4 kW single-phase

  - Fronius Symo 10: 10 kW three-phase

- Sometimes you see decimals like 3.8, 5, 10, 15. These are just the exact rated power of the inverter in kW.

3. Phase designation

- 1P: Single-phase output (typical for PRIMO, residential)

- 3P: Three-phase output (typical for SYMO or ECO three-phase)

4. Communication or feature letters

- LIGHT: Basic model, fewer monitoring features, usually no Wi-Fi

- WLAN: Built-in Wi-Fi for monitoring with Solar.web / Fronius apps

5. Other suffixes

- M: Sometimes seen in Simo 15.0-3-M, can indicate firmware version, monitoring capability, or “Modular” in some documentation. Fronius doesn’t always publicly define this letter.

- -3: Could indicate three-phase AC output in older naming schemes.

Generally:

- All SYMO and most PRIMO models have dynamic voltage control capability.

- ECO series may be limited, especially LIGHT versions.

- SYMO, PRIMO, ECO are grid-tied only models and are not Hybrid.

Manufacture GOODWE

GoodWe inverters generally support dynamic voltage control, including Volt-Var and Volt-Watt functionalities, to comply with modern grid codes and enhance power quality. These features enable inverters to adjust reactive power output in real-time based on grid voltage variations, thereby stabilizing the local grid.

GoodWe inverter families:

- Single-phase models: 1, 2, 3, 5, 6, 6.5, 8.5, 10 kW

- Three-phase models: 3, 5, 6, 8, 10, 15, 20, 25, 30, 36, 50, 60, 80, 100, 110, 120 kW

- Most modern GoodWe single-phase and three-phase inverters support dynamic voltage control according to their grid code compliance (EN50549-1, VDE-AR-N 4105, etc.).

- Typically, this includes:

  - Dynamic voltage regulation (volt-var, volt-watt curves)

  - Reactive power support based on grid voltage fluctuations

- GoodWe inverters generally support dynamic voltage control, but the exact functionality can depend on the firmware version and local grid configuration.

- Hybrid models are usually labeled EH, ES, ET, HT, EM-H, A-ES or DNS-H.

- GoodWe inverters are designed to meet various international grid standards, including VDE-AR-N 4105 and IEC 61727, which often require dynamic voltage control capabilities.

Manufacture Huawei

Huawei SUN2000 inverter models support dynamic voltage control features, which can adjust their output in response to grid voltage variations. These features are typically part of reactive power regulation modes, such as Volt-VAr and Volt-Watt.

Main models:

SUN2000-M1 (residential hybrid), SUN2000-KTL (string), SUN2000-185KTL (utility scale).

All these models support DVC.

Inverters with Dynamic Voltage Control

- Huawei SUN2000-(50, 60, 65)KTL-M0
These commercial-grade inverters support both Volt-VAr (reactive power regulation) and Volt-Watt (active power derating) modes. Volt-Watt mode is enabled by default and adjusts the inverter's output to maintain grid stability during voltage fluctuations.

- Huawei SUN2000-(3, 3.68, 4, 5, 6)KTL-L1
These residential inverters offer reactive power regulation (Volt-VAr mode).

Inverters with no explicit Dynamic Voltage Control

The following inverter models do not explicitly mention support for dynamic voltage control features in their available documentation:

- Huawei SUN2000-8K-LC0
This residential inverter is designed for high efficiency and safety but does not list dynamic voltage control features in its specifications.

- Huawei SUN2000-10K-LC0
Similar to the 8K-LC0 model, this inverter focuses on efficiency and safety without explicit mention of dynamic voltage control capabilities.

- Huawei SUN2000-(5, 8, 10, 12, 20, 100)KTL-M1
While these inverters are part of Huawei's commercial offerings, their documentation does not specify support for dynamic voltage control features.

- Huawei SUN2000-(12, 15, 20, 30)KTL-M2
These inverters are designed for larger residential or commercial applications but do not explicitly mention dynamic voltage control in their available specifications.

- Huawei SUN2000-(30, 40, 50)KTL-M3
This series focuses on high-power applications, but dynamic voltage control features are not detailed in the available documentation.

- Huawei SUN2000-10K-LC1
Similar to the LC0 series, this inverter emphasizes efficiency and safety without explicit support for dynamic voltage control.

- Huawei SUN2000-60KTL-L0
This commercial inverter does not list dynamic voltage control features in its specifications.

- Huawei SUN2000-6KTL-L2
This residential inverter focuses on efficiency and safety but does not mention dynamic voltage control capabilities.

It is important to note that while some models do not explicitly mention dynamic voltage control, they may still comply with local grid codes that require such functionalities.

The following are hybrid inverters, capable of integrating with energy storage systems (ESS) for backup power and enhanced energy management:

Hybrid Huawei SUN2000 Inverters

- Huawei SUN2000-6KTL-L1
A single-phase hybrid inverter with a 6 kW output, designed for residential use. It supports battery integration and features WLAN connectivity for monitoring.

- Huawei SUN2000-8KTL-LC0
A single-phase hybrid inverter offering 8 kW output, suitable for residential applications. It includes AFCI protection and is battery-ready, allowing for future energy storage integration.

- Huawei SUN2000-10K-LC1
A single-phase hybrid inverter with a 10 kW output, designed for residential use. It supports battery integration and features advanced protection mechanisms.

- Huawei SUN2000-3KTL-L1
A single-phase hybrid inverter with a 3 kW output, ideal for smaller residential setups. It is battery-ready and supports smart monitoring features.

- Huawei SUN2000-4KTL-L1
A single-phase hybrid inverter offering 4 kW output, suitable for medium-sized residential applications. It supports battery integration and features smart monitoring capabilities.

- Huawei SUN2000-4.6KTL-L1
A single-phase hybrid inverter with a 4.6 kW output, designed for residential use. It is battery-ready and supports advanced monitoring features.

- Huawei SUN2000-5KTL-L1
A single-phase hybrid inverter offering 5 kW output, suitable for residential applications. It supports battery integration and features smart monitoring capabilities.

- Huawei SUN2000-6K-MAP0
A three-phase hybrid inverter with a 6 kW output, designed for residential use. It supports battery integration and features advanced energy management capabilities.

- Huawei SUN2000-8K-MAP0
A three-phase hybrid inverter offering 8 kW output, suitable for larger residential setups. It supports battery integration and features intelligent energy management.

- Huawei SUN2000-10K-MAP0
A three-phase hybrid inverter with a 10 kW output, designed for residential use. It supports battery integration and offers advanced energy management features.

- Huawei SUN2000-12K-MAP0
A three-phase hybrid inverter offering 12 kW output, suitable for larger residential applications. It supports battery integration and features intelligent energy management.

- Huawei SUN2000-15K-MB0
A three-phase hybrid inverter with a 15 kW output, designed for residential use. It supports battery integration and offers advanced energy management capabilities.

- Huawei SUN2000-20K-MB0
A three-phase hybrid inverter offering 20 kW output, suitable for larger residential setups. It supports battery integration and features intelligent energy management.

- Huawei SUN2000-25K-MB0
A three-phase hybrid inverter with a 25 kW output, designed for residential use. It supports battery integration and offers advanced energy management features.

These hybrid inverters are designed to work seamlessly with Huawei's LUNA2000 battery storage systems, providing efficient energy management and backup power solutions. They are suitable for various residential applications, depending on energy needs and system requirements.

Hybrid Huawei SUN2000 Inverters

Single-Phase Models (2–6 kW)

These inverters are suitable for residential applications and can be paired with the LUNA2000 battery storage system:

- Huawei SUN2000-2KTL-L1

- Huawei SUN2000-3KTL-L1

- Huawei SUN2000-4KTL-L1

- Huawei SUN2000-4.6KTL-L1

- Huawei SUN2000-5KTL-L1

- Huawei SUN2000-6KTL-L1

These models are battery-ready and can be integrated with Huawei's LUNA Smart String Energy Storage System, SmartGuard, and EMMA (Energy Management Assistant

Three-Phase Models (5–25 kW)

These inverters are designed for larger residential or commercial applications and support battery integration:

- Huawei SUN2000-5K-MAP0

- Huawei SUN2000-6K-MAP0

- Huawei SUN2000-8K-MAP0

- Huawei SUN2000-10K-MAP0

- Huawei SUN2000-12K-MAP0

- Huawei SUN2000-15K-MB0

- Huawei SUN2000-20K-MB0

- Huawei SUN2000-25K-MB0

These models feature high efficiency (up to 98.6%) and support LUNA2000 batteries

Non-Hybrid Huawei SUN2000 Inverters

The following models are not hybrid inverters and do not support battery integration:

- Huawei SUN2000-50KTL-M0

- Huawei SUN2000-60KTL-M0

- Huawei SUN2000-100KTL-M1

- Huawei SUN2000-150KTL-MG0

Hybrid Capability and Dynamic Voltage Control of Huawei SUN2000 Inverters

| Model | Hybrid (Battery-Ready) | Dynamic Voltage Control |
| --- | --- | --- |
| SUN2000-8K-LC0 | No | No |
| SUN2000-10K-LC0 | No | No |
| SUN2000-10K-LC1 | No | No |
| SUN2000-60KTL-L0 | No | No |
| SUN2000-6KTL-L2 | No | No |
| SUN2000-50KTL-M0 | No | Yes |
| SUN2000-60KTL-M0 | No | Yes |
| SUN2000-100KTL-M1 | No | Yes |
| SUN2000-150KTL-MG0 | No | Yes |
| SUN2000-3KTL-L1 | Yes | Yes |
| SUN2000-4KTL-L1 | Yes | Yes |
| SUN2000-4.6KTL-L1 | Yes | Yes |
| SUN2000-5KTL-L1 | Yes | Yes |
| SUN2000-6KTL-L1 | Yes | Yes |
| SUN2000-3KTL-M1 | Yes | Yes |
| SUN2000-4KTL-M1 | Yes | Yes |
| SUN2000-5KTL-M1 | Yes | Yes |
| SUN2000-6KTL-M1 | Yes | Yes |
| SUN2000-8KTL-M1 | Yes | Yes |
| SUN2000-10KTL-M1 | Yes | Yes |
| SUN2000-12K-MAP0 | Yes | Yes |
| SUN2000-15K-MAP0 | Yes | Yes |
| SUN2000-20K-MAP0 | Yes | Yes |
| SUN2000-25K-MAP0 | Yes | Yes |
| SUN2000-30K-MAP0 | Yes | Yes |
| SUN2000-36K-MAP0 | Yes | Yes |
| SUN2000-40K-MAP0 | Yes | Yes |
| SUN2000-50K-MAP0 | Yes | Yes |

Manufacure KACO

- Blueplanet 3.0-5.0 TL1

The inverter have static voltage control. It is not certified to VDE-AR-N 4105 or similar. TL1 series not supported hybrid.

- Blueplanet 3.0-10.0 TL3

The inverter have dynamic voltage control and certificate under VDE-AR-N 4105. TL3 series support hybrid.

So the KACO blueplanet inverters such as the 7.5, 8.6, 9, 10 and 20 kW TL3 models, the 5.0 TL1, and the 15.0 TL3 - are primarily designed for grid-tied ASPPs systems. However, only the TL3 models (e.g., 10.0 TL3) are hybrid inverters.

Hybrid and Standard Inverters

- Hybrid Inverters (e.g., blueplanet 10.0 TL3): These inverters integrate solar energy conversion with battery charging and discharging capabilities. They can operate in grid-tied, off-grid or backup modes, making them suitable for systems with energy storage.

- Standard Inverters (e.g., blueplanet 5.0 TL1): These are designed solely for converting DC power from solar panels into AC power for the grid. They do not have the functionality to manage battery storage or operate in off-grid modes.

Manufacture KSTAR

Below listed a wide range of KSG/KeysStar/Blue-G/Blue-S inverters.

1. KSG Series (KeysStar)

| Model | Power | Type | Notes |
| --- | --- | --- | --- |
| KSG-(12,15,20,25,50,60) K | 12–60 kW | Likely 3-phase string inverter | “K” series usually grid-tied, mostly static control (check model spec) |
| KeysStar (5, 10, 15 kW) | 5–15 kW | Single/3-phase | Grid-tied; may have Q(U) reactive control option |
| KSG-(8,10) K-TL | 8–10 kW | String inverter | TL often denotes transformerless; dynamic voltage support? Maybe yes |
| KSG-(10,12,15,20,25,40) KT-M1 | 10–40 kW | String inverter | KT = Kinetic Technology? M1 = single MPPT? Usually dynamic Q(U) support |
| KSG-(4,7) K-DM | 4–7 kW | Single-phase | DM = dual-MPPT; hybrid? Usually not |

2. Blue Series

| Model | Power | Type | Notes |
| --- | --- | --- | --- |
| Blue-(4,5,6,8) KT | 4–8 kW | Single-phase | KT = dynamic control possible |
| G(50,60) KT | 50–60 kW | 3-phase | Likely dynamic voltage control (LVRT/Q(U) curve) |
| Blue-G (2–3 kW) S-(2,3) K | 2–3 kW | Single-phase | Static grid-tied, low power |
| Blue-G (3–8 kW) D-(3–8) K | 3–8 kW | Single-phase | “D” = dual-MPPT, dynamic voltage control supported |
| Blue-Glow (10–12 kW) G-(10–12) K | 10–12 kW | Single-phase | Possibly hybrid; may support battery |
| BLUE-S HYBRID 3ph 10 kW-10.2 kWh | 10 kW / 10.2 kWh | Hybrid, all-in-one | Supports grid, PV, and battery; 3-phase |
| BLUE-S HYBRID 5 kW-10.2 kWh / 5.1 kWh | 5 kW / 5–10.2 kWh | Hybrid, all-in-one | Single-phase or 3-phase options |

3. Other KeysStar / KSG models

| Model | Power | Type | Notes |
| --- | --- | --- | --- |
| G125KT-7 | ~125 kW | 3-phase | High-power string, dynamic Q(U) likely |
| KSG-120CL-M0 | 120 kW | 3-phase | Utility scale, dynamic voltage support |
| KSG-250UH | 250 kW | 3-phase | Large-scale PV, usually LVRT/dynamic control |
| KSG-30K-TL | 30 kW | 2-track (dual MPPT) | TL = transformerless; dynamic voltage support possible |
| KSG-30KT-M1 | 30 kW | 3-track | KT = dynamic control; M1 = single MPPT per track? |

General Notes:

- Hybrid models: Only the BLUE-S HYBRID series is explicitly hybrid (supports batteries). Most KSG and Blue-G are grid-tied only.

- Dynamic voltage control (Q(U), LVRT):

  - Models with “KT”, “TL”, or high-power G-series usually have dynamic voltage control.

  - Low-power “K” or “S” series may only support static PF or voltage control.

- All-in-one solutions: BLUE-S HYBRID comes with PV + inverter + battery in one unit, so fully hybrid.

- High-power / utility-scale: Models like KSG-250UH, KSG-120CL-M0, G125KT-7 are for large PV plants; they support dynamic voltage and reactive power management for grid compliance.

| Model | Power | Type | Hybrid | Dynamic Voltage Control | Notes |
| --- | --- | --- | --- | --- | --- |
| KSG-(12,15,20,25,50,60) K | 12–60 kW | 3-phase | No | Possibly | Grid-tied string inverter |
| KeysStar (5,10,15) | 5–15 kW | Single/3-phase | No | Possibly | Small grid-tied |
| KSG-(8,10) K-TL | 8–10 kW | Single-phase | No | Yes | TL = Transformerless, supports Q(U) |
| KSG-(10,12,15,20,25,40) KT-M1 | 10–40 kW | 3-phase | No | Yes | KT = dynamic voltage control, M1 = MPPT |
| KSG-(4,7) K-DM | 4–7 kW | Single-phase | No | Possibly | DM = dual MPPT |
| Blue-(4,5,6,8) KT | 4–8 kW | Single-phase | No | Yes | Supports dynamic voltage control |
| G(50,60) KT | 50–60 kW | 3-phase | No | Yes | High-power string inverter |
| Blue-G (2–3 kW) S-(2,3) K | 2–3 kW | Single-phase | No | No | Static grid-tied, low power |
| Blue-G (3–8 kW) D-(3–8) K | 3–8 kW | Single-phase | No | Yes | D = dual-MPPT, dynamic support |
| Blue-Glow (10–12 kW) G-(10–12) K | 10–12 kW | Single-phase | Possibly | Yes | May support hybrid/battery |
| BLUE-S HYBRID 3ph 10 kW-10.2 kWh | 10 kW | 3-phase all-in-one | Yes | Yes | PV + battery + inverter |
| BLUE-S HYBRID 5 kW-10.2 kWh | 5 kW | Single/3-phase all-in-one | Yes | Yes | PV + battery + inverter |
| BLUE-S HYBRID 5 kW-5.1 kWh | 5 kW | Single/3-phase all-in-one | Yes | Yes | PV + battery + inverter |
| G125KT-7 | 125 kW | 3-phase | No | Yes | Utility-scale string inverter |
| KSG-120CL-M0 | 120 kW | 3-phase | No | Yes | Large-scale PV plant |
| KSG-250UH | 250 kW | 3-phase | No | Yes | Utility-scale, dynamic voltage support |
| KSG-30K-TL | 30 kW | 2-track (dual MPPT) | No | Yes | TL = Transformerless |
| KSG-30KT-M1 | 30 kW | 3-track | No | Yes | KT = dynamic voltage, M1 = MPPT |

Summary:

- Only the BLUE-S HYBRID series is truly hybrid.

- Most KT, TL, or high-power G-series support dynamic voltage control.

- Lower-power “K” and “S” series are mostly static grid-tied.

Manufacture SAMIL

The comprehensive comparison table for Samil Power inverters across the specified power ratings (2.3 kW, 3 kW, 3.3 kW, 3.6 kW, 4.4 kW, 5 kW, 5.2 kW, 5.3 kW, and 6 kW) is shown below:

Samil Power Inverter Comparison (2.3–6 kW)

| Model | Power Rating | Hybrid | Dynamic Voltage Control | Notes |
| --- | --- | --- | --- | --- |
| SolarRiver 2300TL | 2.3 kW | No | No | Basic grid-tied inverter; no hybrid or dynamic features. |
| SolarRiver 3000TL | 3.0 kW | No | No | Similar to 2300TL; lacks hybrid and dynamic voltage control. |
| SolarRiver 3300TL | 3.3 kW | No | No | Standard grid-tied inverter; no hybrid or dynamic features. |
| SolarRiver 3400TL | 3.6 kW | No | No | No hybrid or dynamic voltage control capabilities. |
| SolarRiver 4400TL | 4.4 kW | No | No | Grid-tied inverter without hybrid features. |
| SolarRiver 5000TL | 5.0 kW | No | No | Lacks hybrid and dynamic voltage control. |
| SolarRiver 5200TL | 5.2 kW | No | No | Standard inverter; no hybrid or dynamic features. |
| SolarRiver 5300TL | 5.3 kW | No | No | Similar to 5200TL; lacks hybrid and dynamic voltage control. |
| SolarRiver 6000TL | 6.0 kW | No | No | No hybrid or dynamic voltage control capabilities. |

Key Observations:

- Hybrid Capability: The listed Samil Power inverters in this range (2.3–6 kW) are not hybrid models. They are primarily designed as grid-tied inverters without energy storage integration.

- Dynamic Voltage Control: These models do not feature dynamic voltage control. They operate within standard voltage ranges and do not actively adjust voltage in response to grid conditions.

- Product Line: The inverters listed are part of the SolarRiver Series, which focuses on grid-tied applications without hybrid or dynamic voltage features.

Based on the specifications and typical features of Samil SolarRiver 2.3–6 kW models:

- These inverters are standard grid-tied inverters.

- They do not provide configurable reactive power control.

- Their power factor is essentially fixed near unity (cos ≈ 1), meaning they cannot be set to operate at leading or lagging power factors like -0.8 to +0.8.

- They do not have a Q(U) curve or advanced voltage/reactive control, so they cannot provide static VAR support.

Manufacturers SMA

SMA Inverter series

| Model | Power Range | Notes |
| --- | --- | --- |
| Sunny Boy 1VL-40 (1.5–6 kW) | 1.5, 2.5, 3, 4, 5, 6 kW | Single-phase grid-tied, residential. |
| Sunny Tripower TL-30 (15–25 kW) | 15, 20, 25 kW | Three-phase, commercial use, dynamic grid support. |
| Sunny Boy 1AV-41 (4.0–6.0 kW) | 4, 5, 6 kW | Advanced grid support, complies with ENTSO-E/NERC codes. |
| Sunny Tripower (5.0–10.0 kW) | 5, 6, 8, 10 kW | Three-phase small commercial/residential. |
| Sunny Boy (2.5, 4 kW) | 2.5, 4 kW | Basic grid-tied, still supports reactive control. |
| SMA STP Core2 (SMP110-60) | 110 kW | Utility/commercial inverter, strong dynamic grid support. |
| Sunny Boy 3.0 1AV-40 | 3 kW | Single-phase residential inverter. |
| Sunny Boy 6 1VL-41 | 6 kW | Updated series with full grid support functions. |
| Sunny Tripower 6.0 STP6.0-3AV-40 | 6 kW | Three-phase, dynamic Q control, flexible PF range. |
| Sunny Tripower CORE / CORE1 / CORE2 | 50–110 kW range | Large commercial inverters, multiple MPPT, advanced dynamic voltage/reactive control. |
| Sunny Highpower PEAK1 | 150 kW | Utility-scale inverter, advanced voltage/reactive grid code compliance. |

- Sunny Tripower / STP / STP TL / TL-X series (3-phase)

  - Modern 3-phase inverters (STP TL / TL-X, Sunny Tripower Core1) support dynamic reactive power control (Q(V)) and dynamic voltage control (VDE-AR-N 4105 / EN50549 compliance).

- Sunny Boy series (1-phase)

  - Recent models (e.g., Sunny Boy 5.0–8.0, 3.0–5.0 TL) support reactive power control and limited voltage control, but older or smaller models may not support full DVC, but support static voltage control.

- Hybrid inverters (Sunny Boy Storage, Sunny Tripower Storage)

  - These generally support voltage/frequency control for grid support, including Q(V) and sometimes P(V) droop.

Manufacturers Sofar

| Model | Power | Phase | Generation / Series | DVC | Hybrid / Battery |
| --- | --- | --- | --- | --- | --- |
| SOFAR 1500TL | 1.5 kW | 1P | TL | Possibly | No |
| SOFAR 1600TL | 1.6 kW | 1P | TL | Yes | No |
| SOFAR 2200TL | 2.2 kW | 1P | TL | Yes | No |
| SOFAR 2200TL-G3 | 2.2 kW | 1P | G3 | Yes | No |
| SOFAR 3KTL-G3 | 3 kW | 1P | G3 | Yes | No |
| SOFAR 3KTLM-G2 | 3 kW | 1P | G2 | Yes | No |
| SOFAR 3KTLM-G2 WITH WIF | 3 kW | 1P | G2 | Yes | Possibly |
| SOFAR 3KTLM-G3 | 3 kW | 1P | G3 | Yes | No |
| SOFAR HYD 3000-EP | 3 kW | 1P | Hybrid EP | Yes | Yes |
| SOFAR 3000TL | 3 kW | 1P | TL | Yes | No |
| SOFAR 3000TL-G3 | 3 kW | 1P | G3 | Yes | No |
| SOFAR 3000TL-G3 WITH WI | 3 kW | 1P | G3 | Yes | Possibly |
| SOFAR 4KTLM-G2 | 4 kW | 1P | G2 | Yes | No |
| SOFAR 4KTLM-G3 | 4 kW | 1P | G3 | Yes | No |
| SOFAR 5KTLM-G2 | 5 kW | 1P | G2 | Yes | No |
| SOFAR 5KTLM-G3 | 5 kW | 1P | G3 | Yes | No |
| SOFAR 5.5KTLX-G3 | 5.5 kW | 1P | G3 | Yes | No |
| SOFAR HYD 5000-EP | 5 kW | 1P | Hybrid EP | Yes | Yes |
| SOFAR ESI 5K-S1 | 5 kW | 1P | TL / Standard | Yes | Yes |
| SOFAR 6KTLM-G2 | 6 kW | 1P | G2 | Yes | No |
| SOFAR 6KTLM-G2 WITH WIF | 6 kW | 1P | G2 | Yes | Possibly |
| SOFAR 6KTLM-G3 | 6 kW | 1P | G3 | Yes | No |
| SOFAR 6.6KTLX-G3 | 6.6 kW | 1P | G3 | Yes | No |
| SOFAR HYD 5500-EP | 5.5 kW | 1P | Hybrid EP | Yes | Yes |
| SOFAR HYD 6000-EP / HYD-6000-EP | 6 kW | 1P | Hybrid EP | Yes | Yes |
| SOFAR 7KTLM-G3 | 7 kW | 1P | G3 | Yes | No |
| SOFAR 7.5KTLM | 7.5 kW | 1P | TL | Yes | No |
| SOFAR 7.5KTLM WITH WIFI | 7.5 kW | 1P | TL | Yes | Possibly |
| SOFAR 7.5KTLM-G2 | 7.5 kW | 1P | G2 | Yes | No |
| SOFAR 7.7KTLM-G3 | 7.7 kW | 1P | G3 | Yes | No |
| SOFAR 8KTLM-G3 | 8 kW | 1P | G3 | Yes | No |
| SOFAR 8.8KTLX-G3 | 8.8 kW | 1P | G3 | Yes | No |
| SOFAR 10 kW | 10 kW | 3P | TL | Possibly | No |
| SOFAR 10KTLM-G3 | 10 kW | 3P | G3 | Yes | No |
| SOFAR 11–30 kW | 11–30 kW | 3P | TL | Possibly | No |
| SOFAR 11KTL-X | 11 kW | 3P | X | Yes | No |
| SOFAR 11KTLX-G3 | 11 kW | 3P | G3 | Yes | No |
| SOFAR 12KTL-X | 12 kW | 3P | X | Yes | No |
| SOFAR 12KTLX-G3 | 12 kW | 3P | G3 | Yes | No |
| SOFAR 15 kW | 15 kW | 3P | TL | Possibly | No |
| SOFAR 15000TL-G2 | 15 kW | 3P | G2 | Yes | No |
| SOFAR 15KTLX-G3 | 15 kW | 3P | G3 | Yes | No |
| SOFAR 20 kW | 20 kW | 3P | TL | Possibly | No |
| SOFAR 20000TL-G2 | 20 kW | 3P | G2 | Yes | No |
| SOFAR 20–33KTL-G | 20–33 kW | 3P | G | Yes | No |
| SOFAR 20KTLX-G3 | 20 kW | 3P | G3 | Yes | No |
| SOFAR 24KTLX-G3 | 24 kW | 3P | G3 | Yes | No |
| SOFAR 25KTLX-G3 | 25 kW | 3P | G3 | Yes | No |
| SOFAR 25000TL-G2 | 25 kW | 3P | G2 | Yes | No |
| SOFAR 30KTLX-G3 | 30 kW | 3P | G3 | Yes | No |
| SOFAR 30000TL | 30 kW | 3P | TL | Yes | No |
| SOFAR 30000TL-G2 | 30 kW | 3P | G2 | Yes | No |
| SOFAR 3300TL-G3 | 3.3 kW | 1P | G3 | Yes | No |
| SOFAR 33KTLX-G3 | 33 kW | 3P | G3 | Yes | No |
| SOFAR 33000TL-G2 | 33 kW | 3P | G2 | Yes | No |
| SOFAR 36KTLX-G3 | 36 kW | 3P | G3 | Yes | No |
| SOFAR 40000TL | 40 kW | 3P | TL | Yes | No |
| SOFAR 40KTLX-G3 | 40 kW | 3P | G3 | Yes | No |
| SOFAR 50KTLX-G3 | 50 kW | 3P | G3 | Yes | No |
| SOFAR 50000TL | 50 kW | 3P | TL | Yes | No |
| SOFAR 50000TL-G2 | 50 kW | 3P | G2 | Yes | No |
| SOFAR 60000TL | 60 kW | 3P | TL | Yes | No |
| SOFAR 60000TL (WITH DC SWITCH + WIFI) | 60 kW | 3P | TL | Yes | Possibly |
| SOFAR 60000TL-G2 | 60 kW | 3P | G2 | Yes | No |
| SOFAR 60KTLX-G3 | 60 kW | 3P | G3 | Yes | No |
| SOFAR 80KTL | 80 kW | 3P | KTL | Yes | No |
| SOFAR 100KTL | 100 kW | 3P | KTL | Yes | No |
| SOFAR 100KTL-LV | 100 kW | 3P | KTL LV | Yes | No |
| SOFAR 100KTLX-G4 | 100 kW | 3P | G4 | Yes | No |
| SOFAR 110KTLX-G4 | 110 kW | 3P | G4 | Yes | No |

1. Power & Phase

- The number at the start usually indicates rated power in kW.

  - Example: 3KTL: 3 kW, 10KTLM: 10 kW, 100KTL: 100 kW.

- Some ranges use “–” or two numbers for models covering multiple powers.

  - Example: 11–30 kW: covers 11 kW to 30 kW.

2. Series / Generation Letters

- TL / KTL / KTLM / KTLX

  - TL: Standard string inverter series.

  - KTL: High-power string inverter (often 10 kW and above).

  - KTLM: TL series with modified features (like WiFi or EMS capability).

  - KTLX: Advanced generation with DVC and optional grid support features.

- X

  - Indicates advanced version / next-generation features, usually includes Dynamic Voltage Control.

- M

  - Often stands for “modified” or “multi-function”, sometimes related to WiFi / monitoring / EMS.

- G2 / G3 / G4

  - Generation indicator:

    - G2: 2nd generation (older).

    - G3: 3rd generation (newer, with DVC on most models).

    - G4: latest generation, often 100 kW+ models with advanced features.

- LV

  - Low Voltage version, usually for regions with lower grid voltage limits.

3. Hybrid / Special Features

- HYD: Hybrid inverter series; supports batteries.

  - Example: HYD 5000-EP is 5 kW hybrid, “EP” is an internal variant code.

- Wi / WITH WIFI / EMS

  - Indicates WiFi module / energy management system, often allowing battery integration in TL/KTL models.

- EP

  - Typically internal engineering variant for hybrid inverters.

- S1

  - Some small units (like ESI 5K-S1) indicate special version / safety or regional variant.

In short:

- Numbers: power (kW)

- TL / KTL / KTLM / KTLX: series and features

- G2 / G3 / G4: generation

- X: advanced version with DVC

- M: modified / multifunction (WiFi / EMS)

- LV: low voltage version

- HYD / EP: hybrid / battery-capable

- Wi / WITH WIFI: communication & energy management module

Manufacturers Solax

| Model | Hybrid | Dynamic Voltage Control |
| --- | --- | --- |
| X1-BOOST X1-5.0-T-D | No | Yes |
| X1-MINI-X1-3.6-S-D | No | Yes |
| X1-SMART X1-8.0-T-D | No | Yes |
| X1-SMART X1-SMT-10K-G2 | No | Yes |
| X1-SMART X1-SMT-10KT-G2 | No | Yes |
| X1-SMART X1-SMT-7K-G2 | No | Yes |
| X1-SMART X1-SMT-8K-G2 | No | Yes |
| X1-SMART-X1-8.0K | No | Yes |
| X1-2.0(MINI) | No | Yes |
| X1-2.0-S-D | No | Yes |
| X1-2.5(MINI) | No | Yes |
| X1-2.5K-S-D | No | Yes |
| X1-3.0(MINI) | No | Yes |
| X1-3.0K-S-D | No | Yes |
| X1-3.0-S-D | No | Yes |
| X1-3.0-T-D | No | Yes |
| X1-3.3K-S-D | No | Yes |
| X1-3.6(MINI) | No | Yes |
| X1-3.6-S-D | No | Yes |
| X1-4.2-T(BOOST) | No | Yes |
| X1-4.6-T(BOOST) | No | Yes |
| X1-5.0-T(BOOST) | No | Yes |
| X1-5.0-T+WIFI | No | Yes |
| X1-5.0-T-D | No | Yes |
| X1-5.5-T-D | No | Yes |
| X1-6.0K (BOOST) | No | Yes |
| X1-6.0-T(BOOST) | No | Yes |
| X1-6.0-T-D | No | Yes |
| X1-7.0K(SMART) | No | Yes |
| X1-8.0K(SMART) | No | Yes |
| X1-8.0-T-D | No | Yes |
| X1-BOOST-3.6K-G4 | No | Yes |
| X1-BOOST-3K-G4 | No | Yes |
| X1-BOOST-4.2K-G4 | No | Yes |
| X1-BOOST-5K-G4 | No | Yes |
| X1-BOOST-6K-G4 | No | Yes |
| X1-HYB-6.0-LV | Yes | Yes |
| X1-HYBRID-6.0-G4 | Yes | Yes |
| X1-HYBRID-7.5-G4 | Yes | Yes |
| X1-IES-6K | Yes | Yes |
| X1-IES-8K | Yes | Yes |
| X1-LITE-10K-LV | Yes | Yes |
| X1-LITE-8K-LV | Yes | Yes |
| X1-MINI-2.0K-G4 | No | Yes |
| X1-MINI-3.0K-G4 | No | Yes |
| X1-MINI-3.3K-G4 | No | Yes |
| X1-MINI-3.6 | No | Yes |
| X1-MINI-3.7K-G4 | No | Yes |
| X1-MINI-4.0K-G4 | No | Yes |
| X1-SMT-10K-G2 | No | Yes |
| X1-SMT-7K-G2 | No | Yes |
| X1-SMT-8K-G2 | No | Yes |
| X1-VAST-10K | No | Yes |
| X1-VAST-8K | No | Yes |
| X3-10K-TL+WIFI | No | Yes |
| X3-15.0P-T-D | No | Yes |
| X3-15K-TL+WIFI | No | Yes |
| X3-20K-TL+WIFI | No | Yes |
| X3-5.0-T+WIFI | No | Yes |
| X3-50K TL | No | Yes |
| X3-60K TL | No | Yes |
| X3-FTH-100K | No | Yes |
| X3-FTH-125K(12DX) | No | Yes |
| X3-FTH-80K | No | Yes |
| X3-HYBRID-10.0-G4 | Yes | Yes |
| X3-HYBRID-10-G4 | Yes | Yes |
| X3-HYBRID-15.0-G4 | Yes | Yes |
| X3-MGA-40K | No | Yes |
| X3-MGA-40K-G2 | No | Yes |
| X3-MGA-50K | No | Yes |
| X3-MGA-50K-G2 | No | Yes |
| X3-MGA-60K | No | Yes |
| X3-MGA-60K-G2 | No | Yes |
| X3-MIC-10.0K-G2 | No | Yes |
| X3-MIC-10.0K-T-D | No | Yes |
| X3-MIC-10.0-T-D | No | Yes |
| X3-MIC-10K-G2 | No | Yes |
| X3-MIC-12.0K-G2 | No | Yes |
| X3-MIC-12.0K-T-D | No | Yes |
| X3-MIC-12K | No | Yes |
| X3-MIC-15.0K-G2 | No | Yes |
| X3-MIC-15.0K-T-D | No | Yes |
| X3-MIC-15K | No | Yes |
| X3-MIC-15K-G2 | No | Yes |
| X3-MIC-3K | No | Yes |
| X3-MIC-3K-G2 | No | Yes |
| X3-MIC-4K | No | Yes |
| X3-MIC-4K-G2 | No | Yes |
| X3-MIC-5.0K-G2 | No | Yes |
| X3-MIC-5.0K-T-D | No | Yes |
| X3-MIC-5K-G2 | No | Yes |
| X3-MIC-6.0K-G2 | No | Yes |
| X3-MIC-6.0K-T-D | No | Yes |
| X3-MIC-6K | No | Yes |
| X3-MIC-6K-G2 | No | Yes |
| X3-MIC-8.0K-G2 | No | Yes |
| X3-MIC-8.0K-T-D | No | Yes |
| X3-MIC-8K | No | Yes |
| X3-MIC-8K-G2 | No | Yes |
| X3-NEO-10K-LV | Yes | Yes |
| X3-NEO-12K-LV | Yes | Yes |
| X3-NEO-15K-LV | Yes | Yes |
| X3-PRO G1.1 | No | Yes |
| X3-PRO-10K | No | Yes |
| X3-PRO-10K-G2 | No | Yes |
| X3-PRO-10K-G2(2D) | No | Yes |
| X3-PRO-12K-G2 | No | Yes |
| X3-PRO-15K-G2 | No | Yes |
| X3-PRO-15K-G2(2D) | No | Yes |
| X3-PRO-17K-G2 | No | Yes |
| X3-PRO-20K-G2 | No | Yes |
| X3-PRO-20K-G2(2D) | No | Yes |
| X3-PRO-25K-G2 | No | Yes |
| X3-PRO-25K-G2(3D) | No | Yes |
| X3-PRO-30.0K-R-D | No | Yes |
| X3-PRO-30K | No | Yes |
| X3-PRO-30K-G2 | No | Yes |
| X3-PRO-8K-G2 | No | Yes |
| X3-ULT-15K | No | Yes |
| X3-ULT-20K | No | Yes |
| X3-ULT-25K | No | Yes |
| X3-ULT-30K | No | Yes |
| XI-BOOST-5K-G4 | No | Yes |

General Prefix

- X1: Single-phase

- X3: Three-phase

Series / Family

- MINI: Small single-phase (1.5–3.6 kW), compact entry-level

- BOOST: Mid-range single-phase (3–6 kW), higher efficiency

- SMART / SMT: Advanced single-phase, bigger power range (up to ~10 kW)

- MIC: Entry-level three-phase (3–15 kW)

- PRO: High-end three-phase commercial (8–30 kW)

- ULT (Ultra): Larger commercial 15–30 kW

- MGA: Utility / large commercial (40–60 kW)

- FTH: Large-scale 80–125 kW

- VAST: Newer series, single-phase, higher output (8–10 kW)

- HYBRID / HYB / NEO / IES / LITE: Hybrid models (with battery support)

Power Rating

- Numbers (e.g., 3.0K, 5K, 10K, 15K) = Nominal output power in kW

- Decimal format sometimes used (e.g., 15.0K = 15 kW)

Suffixes / Letters

- T: Twin MPPT trackers

- TL: Transformerless

- D: With DC switch

- P: Parallel version (sometimes “Pro” series variant)

- R: Reinforced / revised design

- G1, G2, G3, G4: Product generation (newer = more features, better efficiency, firmware updates)

- LV: Low Voltage (battery compatibility for hybrids)

- +WIFI =Built-in WiFi module

- K: Sometimes shorthand for kW rating (e.g., 5K = 5 kW)

Dynamic Voltage Control

- All models support DVC (because all SolaX inverters comply with EN50549 / VDE-4105 requirements).

Hybrid Models (Battery-capable)

Single-phase (X1)

- X1-HYB-6.0-LV

- X1-HYBRID-6.0-G4

- X1-HYBRID-7.5-G4

- X1-IES-6K

- X1-IES-8K

- X1-LITE-8K-LV

- X1-LITE-10K-LV

Three-phase (X3)

- X3-HYBRID-10.0-G4

- X3-HYBRID-10-G4

- X3-HYBRID-15.0-G4

- X3-NEO-10K-LV

- X3-NEO-12K-LV

- X3-NEO-15K-LV

Non-Hybrid Models (Grid-tied only)

- All others in listed above (BOOST, MINI, SMART, SMT, VAST, MIC, MGA, PRO, ULT, FTH, etc.) are grid-tied only.

- They still have DVC, but cannot connect batteries directly.

Manufacturers Solis

| Model | DVC | Hybrid |
| --- | --- | --- |
| SOLIS 100K-5G | Yes | No |
| SOLIS 100K-5G-PRO | Yes | No |
| SOLIS 10KW-3P | Yes | No |
| SOLIS 110K-5G | Yes | No |
| SOLIS 110K-5G-PRO | Yes | No |
| SOLIS 1P10K | Yes | No |
| SOLIS 1P10K-4G | Yes | No |
| SOLIS 1P3K-4G | Yes | No |
| SOLIS 1P4K-4G | Yes | No |
| SOLIS 1P5K-4G | Yes | No |
| SOLIS 1P5K-4 | Yes | No |
| SOLIS 1P8K | Yes | No |
| SOLIS 1P9K | Yes | No |
| SOLIS 250K-EHV-5G | Yes | No |
| SOLIS 250K-EHV-5G-PLUS | Yes | No |
| SOLIS 3P100K | Yes | No |
| SOLIS 3P100K-5G-PRO | Yes | No |
| SOLIS 3P80K-5G-PRO | Yes | No |
| SOLIS 3P10K | Yes | No |
| SOLIS 3P10K-4G | Yes | No |
| SOLIS 3P12K | Yes | No |
| SOLIS 3P12K-4G | Yes | No |
| SOLIS 3P15K | Yes | No |
| SOLIS 3P15K-4G | Yes | No |
| SOLIS 3P20K | Yes | No |
| SOLIS 3P20K-4G | Yes | No |
| SOLIS 3P25K | Yes | No |
| SOLIS 3P30K | Yes | No |
| SOLIS 46K | Yes | No |
| SOLIS 50K-5G | Yes | No |
| SOLIS 5K | Yes | No |
| SOLIS 60K-4G | Yes | No |
| SOLIS 6-GR1P8K2 | Yes | No |
| SOLIS 6K | Yes | No |
| SOLIS 7K | Yes | No |
| SOLIS 80K-5G-PRO | Yes | No |
| SOLIS G5-GC60K-M12-3-DC60 | Yes | No |
| SOLIS G6-GC50K | Yes | No |
| SOLIS K-5G-PRO 100KW | Yes | No |
| SOLIS MINI-2500-4G | Yes | No |
| SOLIS MINI-3000-4G | Yes | No |
| SOLIS S5-GC25K / 30K / 33K / 36K / 40K / 50K / 60K | Yes | No |
| SOLIS S5-GC3P30K / 50K / 100K | Yes | No |
| SOLIS S5-GR1P1 / 7K / 8K / 10K | Yes | No |
| SOLIS S5-GR3P1 / 8K / 10K / 15K / 17K / 20K | Yes | No |
| SOLIS S5-GRIP10K / S5-GRP3P10K | Yes | No |
| SOLIS S6-EH1P5K-L-EU | Yes | Yes |
| SOLIS S6-EH1P5K-L-PLUS | Yes | Yes |
| SOLIS S6-EH1P8K-L-PRO | Yes | Yes |
| SOLIS S6-EH1P12K03-NV-YD-L | Yes | Yes |
| SOLIS S6-EH3P8K02-NV-YD-L | Yes | Yes |
| SOLIS S6-EH3P10K02-NV-YD-L | Yes | Yes |
| SOLIS S6-EH3P15K02-NV-YD-L | Yes | Yes |
| SOLIS S6-GC100K / 110K | Yes | No |
| SOLIS S6-GR1P2K-M / 3K-M / 3K-S / 4K / 4K-S / 5K / 5K-S / 6K / 6K-S / 7K / 7K2 / 8 / 8K / 8K2 | Yes | No |
| SOLIS S6-GU350K-EHV-M12 | Yes | No |
| SOLIS X1-BOOST-3.6K-G4 | Yes | No |
| SOLIS X1-MINI-2.0K / 3.0K / 4K (G4) | Yes | No |

Summary:

- All Solis inverters support Dynamic Voltage Control

- Only S6-EH models support Hybrid (battery)

Solis Model Name Structure

Typical format:
SOLIS [Series/Generation] – [Phase/Power] – [Extra Features/Region]

Meaning of Letters & Codes

- 1P: Single-phase inverter

- 3P: Three-phase inverter

- K / KW: Nominal power rating in kilowatts (e.g., 10K = 10 kW)

- MINI: Small residential models (low-power string inverters)

- BOOST: Entry-level single-phase inverter series

- EH: Hybrid energy storage inverter (can connect to batteries)

- GC: Commercial / Utility scale inverters (large capacity)

- GR: Residential series

- S5, S6, 4G, 5G, G4, G5, G6: Generation/version of design

  - 4G = 4th Gen

  - 5G = 5th Gen

  - 6G / S6 = 6th Gen (latest)

- PRO / PLUS: Enhanced features (higher efficiency, advanced comms, better grid support)

- EHV: Extra High Voltage (for large-scale solar farms)

- X1: Single-phase series (Boost/Mini)

- M or –S: Variant models (slight changes in design, region, display, or comms)

- NV-YD-L / EU: Market-specific versions (e.g., EU = Europe)

Main Categories in Listed above

- Residential small-scale (1–10 kW)

  - MINI, BOOST, X1, GR1P, S6-GR1P

  - Examples: SOLIS 1P5K-4G, SOLIS X1-MINI-3.0K-G4

- Residential / Small Commercial 3-phase (10–30 kW)

  - GR3P, S5-GR3P, S6-GR3P

  - Examples: SOLIS 3P20K-4G, SOLIS S5-GR3P15K

- Commercial Medium (30–100 kW)

  - S5-GC, S6-GC, 3P PRO series

  - Examples: SOLIS 50K-5G, SOLIS S6-GC100K

- Utility / Large-scale (>100 kW)

  - GC Large, EHV

  - Examples: SOLIS 250K-EHV-5G, SOLIS S6-GU350K-EHV-M12

- Hybrid (with battery support)

  - EH series

  - Examples: SOLIS S6-EH1P5K-L-EU, SOLIS S6-EH3P15K02-NV-YD-L

Example: SOLIS S6-EH3P10K02-NV-YD-L:

- S6 = 6th Gen

- EH = Hybrid

- 3P = Three-phase

- 10K = 10 kW

- 02 = Model version

- NV-YD-L = Region/certification code